2023 Spring VLSI DSP Homework Assignment #3

4108064101 杜冠廷

- Determining the word length of the filter coefficients

Matlab code:

Computing the filter coefficients difference between floating point and fixed point

clear all close all clc %Filter coefficients h = [ 0.037828455507; -0.023849465020; -0.110624404418; 0.377402855613; 0.852698679009; 0.377402855613; -0.110624404418; -0.023849465020; 0.037828455507]; g = [-0.064538882629; 0.040689417609; 0.418092273222; -0.788485616406; 0.418092273222; 0.040689417609; -0.064538882629]; W = 16; h_error = zeros(W, 1); g_error = zeros(W, 1); for i = 8 : W h_fix = fi(h, 1, i, i-1); g_fix = fi(g, 1, i, i-1); h_error(i) = sum(abs(h - double(h_fix))) * 100; g_error(i) = sum(abs(g - double(g_fix))) * 100; end figure(1) hold on plot(h_error,'k*-'); plot(g_error,'bs-'); legend('h', 'g'); xlabel('coefficient word length'); ylabel('Coefficient difference (%)'); xlim([8 W]); ylim([0 2.0]); title('Filter coefficients difference'); hold off

Result:

Choosing the word length of the filter coefficients are 11.

- Determining the word length of the filter outputs at each level

Matlab code:

clear all close all clc %Read image test_image = fi(imread('HW3_test_image.bmp') ,0 ,8 ,0); %Define output result matrix DWT_out = zeros(512,512); %Define image process length L = 512; %Define fixed point precision layer1_fixed = numerictype(true, 10, 0); % max : 504 min : -223 width(min.) = f + 9 + 1 (f == fraction number) layer2_fixed = numerictype(true, 11, 0); % max : 985 min : -295 width(min.) = f + 10 + 1 (f == fraction number) layer3_fixed = numerictype(true, 12, 0); % max : 1952 min : -469 width(min.) = f + 11 + 1 (f == fraction number) %DWT [out_l ,out_h] = DWT_row_processing(L ,test_image ,layer1_fixed); [out_ll ,out_hl ,out_lh ,out_hh] = DWT_column_processing(L ,out_l ,out_h ,layer1_fixed); DWT_out(1 : 256 ,257 : 512) = out_hl; DWT_out(257 : 512 ,1 : 256) = out_lh; DWT_out(257 : 512 ,257 : 512) = out_hh; [out_l_2 ,out_h_2] = DWT_row_processing(256 ,out_ll ,layer2_fixed); [out_ll_2 ,out_hl_2 ,out_lh_2 ,out_hh_2] = DWT_column_processing(256 ,out_l_2 ,out_h_2 ,layer2_fixed); DWT_out(1 : 128 ,129 : 256) = out_hl_2; DWT_out(129 : 256 ,1 : 128) = out_lh_2; DWT_out(129 : 256 ,129 : 256) = out_hh_2; [out_l_3 ,out_h_3] = DWT_row_processing(128 ,out_ll_2 ,layer3_fixed); [out_ll_3 ,out_hl_3 ,out_lh_3 ,out_hh_3] = DWT_column_processing(128 ,out_l_3 ,out_h_3 ,layer3_fixed); DWT_out(1 : 64 ,1 : 64) = out_ll_3; DWT_out(1 : 64 ,65 : 128) = out_hl_3; DWT_out(65 : 128 ,1 : 64) = out_lh_3; DWT_out(65 : 128 ,65 : 128) = out_hh_3; %IDWT [I_out_l_2 ,I_out_h_2] = IDWT_column_processing(128 ,out_ll_3 ,out_lh_3 ,out_hl_3 ,out_hh_3); [inv_pic_2] = IDWT_row_processing(128 ,I_out_l_2 ,I_out_h_2); [I_out_l_1 ,I_out_h_1] = IDWT_column_processing(256 ,inv_pic_2 ,out_lh_2 ,out_hl_2 ,out_hh_2); [inv_pic_1] = IDWT_row_processing(256 ,I_out_l_1 ,I_out_h_1); [I_out_l ,I_out_h] = IDWT_column_processing(L ,inv_pic_1 ,out_lh ,out_hl ,out_hh); [inv_pic] = IDWT_row_processing(L ,I_out_l ,I_out_h); %PSNR MSE = 0; for i = 1 : 512 for j = 1 : 512 MSE = MSE + ((double(test_image(i ,j)) - double(inv_pic(i ,j))) ^ 2); end end MSE = MSE / (512 ^ 2); PSNR = 10 * (log10((255 ^ 2) / MSE)); disp(['PSNR = ',num2str(PSNR) ,' dB']); %Display image after processing figure(1) imshow(mat2gray(double(test_image))); figure(2) imshow(mat2gray(double(inv_pic)));

function [out_l ,out_h] = DWT_row_processing(L ,pic ,fixed_size) %Filter coefficients h_floating = [ 0.037828455507; -0.023849465020; -0.110624404418; 0.377402855613; 0.852698679009; 0.377402855613; -0.110624404418; -0.023849465020; 0.037828455507]; g_floating = [-0.064538882629; 0.040689417609; 0.418092273222; -0.788485616406; 0.418092273222; 0.040689417609; -0.064538882629]; %Fixed point coefficients h = fi(h_floating ,1 ,11 ,10); g = fi(g_floating ,1 ,11 ,10); %Symmetric extension at picture boundary p_l = zeros(L ,L + 8 ,'like', fi([], fixed_size)); p_l = [pic( : ,5) pic( : ,4) pic( : ,3) pic( : ,2) pic pic( : ,L - 1) pic( : ,L - 2) pic( : ,L - 3) pic( : ,L - 4)]; p_h = zeros(L ,L + 6 ,'like', fi([], fixed_size)); p_h = [pic( : ,4) pic( : ,3) pic( : ,2) pic pic( : ,L - 1) pic( : ,L - 2) pic( : ,L - 3)]; %Compute output picture for i = 1 : L %Lowpass filter temp_l = conv(p_l(i , :) ,h); out_l(i , 1 : (L / 2)) = fi(temp_l(1 ,9 : 2 : (L + 7)) ,fixed_size); %Highpass filter temp_h = conv(p_h(i , :) ,g); out_h(i , 1 : (L / 2)) = fi(temp_h(1 ,8 : 2 : (L + 6)) ,fixed_size); end end

function [out_ll ,out_hl ,out_lh ,out_hh] = DWT_column_processing(L ,input_l ,input_h ,fixed_size) %Filter coefficients h_floating = [ 0.037828455507; -0.023849465020; -0.110624404418; 0.377402855613; 0.852698679009; 0.377402855613; -0.110624404418; -0.023849465020; 0.037828455507]; g_floating = [-0.064538882629; 0.040689417609; 0.418092273222; -0.788485616406; 0.418092273222; 0.040689417609; -0.064538882629]; %Fixed point coefficients h = fi(h_floating ,1 ,11 ,10); g = fi(g_floating ,1 ,11 ,10); %Symmetric extension at picture boundary input_l_extension_for_l = zeros(L + 8 ,L / 2 ,'like', fi([], fixed_size)); input_l_extension_for_l = [input_l(5 , : ); input_l(4 , : ); input_l(3 , : ); input_l(2 , : ); input_l; input_l(L - 1 , : ); input_l(L - 2 , : ); input_l(L - 3, : ); input_l(L - 4, : )]; input_l_extension_for_h = zeros(L + 6 ,L / 2 ,'like', fi([], fixed_size)); input_l_extension_for_h = [input_l(4 , : ); input_l(3 , : ); input_l(2 , : ); input_l; input_l(L - 1 , : ); input_l(L - 2 , : ); input_l(L - 3, : )]; input_h_extension_for_l = zeros(L + 8 ,L / 2 ,'like', fi([], fixed_size)); input_h_extension_for_l = [input_h(5 , : ); input_h(4 , : ); input_h(3 , : ); input_h(2 , : ); input_h; input_h(L - 1 , : ); input_h(L - 2 , : ); input_h(L - 3, : ); input_h(L - 4, : )]; input_h_extension_for_h = zeros(L + 6 ,L / 2 ,'like', fi([], fixed_size)); input_h_extension_for_h = [input_h(4 , : ); input_h(3 , : ); input_h(2 , : ); input_h; input_h(L - 1 , : ); input_h(L - 2 , : ); input_h(L - 3, : )]; %Compute output picture for i = 1 : L/2 %Lowpass filter temp_ll = conv(input_l_extension_for_l( : ,i) ,h); out_ll(1 : (L / 2) ,i) = fi(temp_ll(9 : 2 : (L + 7) ,1) ,fixed_size); temp_hl = conv(input_h_extension_for_l( : ,i) ,h); out_hl(1 : (L / 2) ,i) = fi(temp_hl(9 : 2 : (L + 7) ,1) ,fixed_size); %Highpass filter temp_lh = conv(input_l_extension_for_h( : ,i) ,g); out_lh(1 : (L / 2) ,i) = fi(temp_lh(8 : 2 : (L + 6) ,1) ,fixed_size); temp_hh = conv(input_h_extension_for_h( : ,i) ,g); out_hh(1 : (L / 2) ,i) = fi(temp_hh(8 : 2 : (L + 6) ,1) ,fixed_size); end end

IDWT_column_processing function and IDWT_row_processing function are the same as HW2 assignment.

Result:

The word length of the filter outputs at layer 1: Data Type Mode; Fixed-point: binary point scaling. Signed or unsigned; Signed. Word Length; 10. Sign bit Length; 1. Integer Length; 9. Fraction Length; 0. The word length of the filter outputs at layer 2: Data Type Mode; Fixed-point: binary point scaling. Signed or unsigned; Signed. Word Length; 11. Sign bit Length; 1. Integer Length; 10. Fraction Length; 0. The word length of the filter outputs at layer 3: Data Type Mode; Fixed-point: binary point scaling. Signed or unsigned; Signed. Word Length; 12. Sign bit Length; 1. Integer Length; 11. Fraction Length; 0.

- The area of each multiplier, adder, and register

Verilog code:

Multiplier

Adder

Register

Result:

| Multiplier |  |  |
| --- | --- | --- |
| Layer 1 |  |  |
| Layer 2 |  |  |
| Layer 3 |  |  |
| Adder |  |  |
| Layer 1 |  |  |
| Layer 2 |  |  |
| Layer 3 |  |  |
| Register |  |  |
| 1 bit register |  |  |
| Layer 1 output |  | 29×10=290 μm2/per pixel |
| Layer 2 output |  | 29×11=319 μm2/per pixel |
| Layer 3 output |  | 29×12=348 μm2/per pixel |

Note:

| Cell Library | Cell-Based Design Kit for IC Contest 製程: TSMC 0.13um 廠商: ARM Design Kit版本：CBDK_IC_Contest_v2.5 |
| --- | --- |
| Synthesis tool | Design Compiler 廠商: Synopsys |

Acknowledgment

NCHU EE Undergraduate				Chun-Wei Su

NCHU EE ICs & Systems Lab 612		Shun-Liang Yeh